# SIMATS ENGINEERING

## ASSESSMENT TOOL 3

### COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

### NAME:P.HEMANTH REDDY

### Reg.No:192312173

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30% Software:Cisco Packet Tracer,Wireshark

Annexure A

## Debugging & Optimisation Task

- A network administrator has configured a workstation with the following settings:

  - IP Address: 192.168.10.65

  - Subnet Mask: 255.255.255.192 (/26)

  - Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

  - Identifying the subnet to which the configured IP address belongs.

  - Determining the network address, broadcast address, and valid host range.

  - Verifying whether the configured IP address and gateway are valid.

  - Identifying the configuration errors.

  - Optimizing the configuration by assigning an appropriate IP address and default gateway.

  - Calculating the total number of usable host addresses after correction.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

- An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

  - Identifying the network address, broadcast address, and host range for each subnet.

  - Calculating the number of usable and unused IP addresses.

  - Identifying inefficient subnet allocations.

  - Debugging the addressing plan by locating subnet allocation mistakes.

  - Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails. Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

- A router contains the following routing table.

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Users report that packets destined for 192.168.2.100 are not reaching the intended network. Task

Debug the routing configuration by:

  - Identifying the matching routing entry using the longest-prefix match.

  - Determining the next-hop router selected for packet forwarding.

  - Verifying whether the destination IP belongs to the intended subnet.

  - Identifying any routing configuration errors.

  - Optimizing the routing table to improve packet forwarding efficiency.

  - Identifying the route that will be used if no matching destination exists.

ANSWERS

1. IPv4 Workstation Configuration Debugging

Given

- IP Address = 192.168.10.65

- Subnet Mask = 255.255.255.192 (/26)

- Default Gateway = 192.168.10.1

The workstation should communicate with systems in subnet:

192.168.10.0/26

Step 1: Identify the Subnet

For /26:

Subnet size = 64

Subnets are:

| Subnet | Address Range |
| --- | --- |
| 192.168.10.0/26 | 0–63 |
| 192.168.10.64/26 | 64–127 |
| 192.168.10.128/26 | 128–191 |
| 192.168.10.192/26 | 192–255 |

IP Address:

192.168.10.65

belongs to:

192.168.10.64/26

Step 2: Determine Network Details

For Subnet 192.168.10.64/26

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.10.64 |
| First Host | 192.168.10.65 |
| Last Host | 192.168.10.126 |
| Broadcast Address | 192.168.10.127 |

Step 3: Verify IP and Gateway

IP Address

192.168.10.65

✔ Valid host address

Gateway

192.168.10.1

Gateway belongs to:

192.168.10.0/26

❌ Different subnet

Therefore the workstation cannot reach its gateway directly.

Step 4: Configuration Error

Error:

- Host is in subnet 192.168.10.64/26

- Gateway is in subnet 192.168.10.0/26

Host and gateway must belong to the same subnet.

Step 5: Optimized Configuration

Option 1 (Recommended)

Keep workstation IP.

Assign gateway within same subnet.

| Parameter | Value |
| --- | --- |
| IP Address | 192.168.10.65 |
| Gateway | 192.168.10.66 |

(or any valid host between 65–126)

Option 2

Keep gateway as 192.168.10.1

Move workstation into subnet 192.168.10.0/26

Example:

IP = 192.168.10.10

Gateway = 192.168.10.1

Step 6: Total Usable Hosts

Host bits:

32 − 26 = 6

Usable hosts:

26-2
64-2=62

Final Answer

- Host belongs to subnet 192.168.10.64/26

- Network Address = 192.168.10.64

- Broadcast Address = 192.168.10.127

- Host Range = 192.168.10.65 – 192.168.10.126

- Error: Gateway is in different subnet

- Correct Gateway Example = 192.168.10.66

- Usable Hosts = 62

2. IPv6 Debugging

Given

Device A:

2001:db8::ff00:42:8329/64

Device B:

2001:db8:0:0:1::1/64

Step 1: Expand IPv6 Addresses

Device A

2001:db8::ff00:42:8329

Expanded:

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:db8:0:0:1::1

Expanded:

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Determine Network Prefix

Prefix Length = /64

First four blocks represent network.

Device A

Network Prefix:

2001:0db8:0000:0000::/64

Device B

Network Prefix:

2001:0db8:0000:0000::/64

Step 3: Same Subnet Verification

Both have identical first 64 bits.

✔ Same subnet

Step 4: Possible Cause of Failure

Addressing is correct.

Possible issues:

- Incorrect default gateway

- Firewall restrictions

- Missing Neighbor Discovery

- Duplicate Address Detection failure

- Interface configuration issue

Step 5: Optimized IPv6 Configuration

Example:

| Device | IPv6 Address |
| --- | --- |
| A | 2001:db8::10/64 |
| B | 2001:db8::20/64 |

Both remain in same subnet.

Step 6: Available Host Addresses

Host bits:

128 − 64 = 64

Available Hosts:

264
18,446,744,073,709,551,616

Final Answer

| Item | Result |
| --- | --- |
| Device A Prefix | 2001:db8::/64 |
| Device B Prefix | 2001:db8::/64 |
| Same Subnet | Yes |
| Likely Issue | Configuration/ND/Gateway problem |
| Available Hosts | 2⁶⁴ |

3. IPv4 Subnet Allocation Optimization

Given Network

192.168.1.0/24

Subnets:

- /26

- /27

- /28

Subnet Details

Subnet 1 (/26)

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.0 |
| Host Range | 192.168.1.1–62 |
| Broadcast | 192.168.1.63 |
| Usable Hosts | 62 |

Subnet 2 (/27)

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.64 |
| Host Range | 192.168.1.65–94 |
| Broadcast | 192.168.1.95 |
| Usable Hosts | 30 |

Subnet 3 (/28)

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.96 |
| Host Range | 192.168.1.97–110 |
| Broadcast | 192.168.1.111 |
| Usable Hosts | 14 |

Usable and Unused Addresses

| Subnet | Usable Hosts |
| --- | --- |
| /26 | 62 |
| /27 | 30 |
| /28 | 14 |

Total Used:

62 + 30 + 14 = 106

Network Capacity:

254 usable hosts

Unused:

254 − 106 = 148

Inefficient Allocation

Problems:

- Small departments may receive /26 unnecessarily.

- Large departments may receive /28 causing shortages.

- Address space wasted.

Optimized VLSM Design

Example Requirements:

| Department | Hosts Needed | Recommended |
| --- | --- | --- |
| Dept A | 50 | /26 |
| Dept B | 25 | /27 |
| Dept C | 10 | /28 |

Allocate largest subnet first.

This minimizes waste and improves scalability.

Final Answer

- Subnet allocations should follow actual host requirements.

- VLSM reduces unused addresses.

- Allocate largest departments first.

- Avoid assigning large subnets to small departments.

4. IPv6 Debugging (Repeated Question)

Expanded Addresses

Device A:

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B:

2001:0db8:0000:0000:0001:0000:0000:0001

Network Prefix

Both:

2001:db8::/64

Same Subnet?

✔ Yes

Possible Issue

- Gateway misconfiguration

- Firewall filtering

- Interface disabled

- Neighbor Discovery issue

Optimized Addresses

- Device A → 2001:db8::10/64

- Device B → 2001:db8::20/64

Available Hosts

264=18,446,744,073,709,551,616

5. Routing Table Debugging

Given Routing Table

| Destination | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Destination Packet:

192.168.2.100

Step 1: Longest Prefix Match

Destination:

192.168.2.100

Matches:

192.168.2.0/24

✔ Exact match found

Step 2: Selected Next Hop

Next Hop:

10.0.0.3

Step 3: Verify Destination Subnet

Network:

192.168.2.0/24

Host Range:

192.168.2.1 – 192.168.2.254

Destination:

192.168.2.100

✔ Valid host in subnet

Step 4: Possible Routing Errors

If communication fails:

- Next-hop router 10.0.0.3 unreachable.

- Interface toward next hop down.

- Missing return route.

- Incorrect subnet mask.

- Routing table not updated.

Step 5: Routing Optimization

Possible improvements:

- Use route summarization.

- Add dynamic routing (OSPF/RIP/EIGRP).

- Monitor next-hop availability.

- Configure backup routes.

Step 6: Route Used if No Match Exists

Default Route:

| Destination | Next Hop |
| --- | --- |
| 0.0.0.0/0 | 10.0.0.1 |

Final Answer

| Task | Result |
| --- | --- |
| Matching Route | 192.168.2.0/24 |
| Longest Prefix Match | 192.168.2.0/24 |
| Next Hop | 10.0.0.3 |
| Destination Valid? | Yes |
| Possible Error | Unreachable next hop / missing return path |
| Default Route | 0.0.0.0/0 → 10.0.0.1 |

### RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |

| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| --- | --- | --- | --- | --- | --- |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structure d code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |