# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

COURSE NAME: CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I “P. HEMANTH REDDY (192312173)” certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

- An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

- A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

- A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead

transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

- A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

ANSWERS:

# Question 1

Given: File=150 MB, Segment size=1500 bytes, Bandwidth=50 Mbps, DL overhead=40 bytes/frame.
Segments=150,000,000/1500=100,000.
Frames=100,000.
Total overhead=100,000×40=4,000,000 bytes=4 MB.
Total transmitted data=154 MB.
Transmission time= (154,000,000×8)/50,000,000=24.64 s.
Answer: 100,000 segments, 100,000 frames, 4 MB overhead, 24.64 s. Transport layer provides segmentation, sequencing and error recovery. Data Link layer provides framing, error detection and reliable node-to-node delivery.

# Question 2

Given: Window size=6, Frame size=1024 bytes, Total frames=48.
Windows required=48/6=8.
One frame lost in each window.
Retransmissions=8×1=8.
Answer: 8 windows and 8 retransmissions. Flow control prevents congestion and receiver overflow.

# Question 3

Given: Packet=12,000 bytes, MTU=1500 bytes, IP header=20 bytes.
Payload/fragment=1500−20=1480 bytes.
Fragments=ceil(12000/1480) =9.
Header overhead=9×20=180 bytes.
Answer: 9 fragments and 180 bytes overhead. Network layer performs routing, fragmentation and reassembly.

# Question 4

Given: Window size=6, Total frames=48.
Windows=48/6=8.
Retransmissions=8.
Answer: 8 transmission windows and 8 retransmissions. Flow control balances sender and receiver speeds.

# Question 5

Given: File=600 MB, checkpoint every 60 MB, Failure after 390 MB.
Checkpoints before failure=360/60=6.
Last checkpoint=360 MB.
Data to retransmit=390−360=30 MB.
Answer: 6 checkpoints and 30 MB retransmission. Session layer synchronization enables recovery from failures.

# Question 6

Given: Original file=900 MB, Compression=40%, Encryption overhead=5%.
Compressed size=900×0.60=540 MB.
Encrypted size=540×1.05=567 MB.
Reduction= ((900−567)/900) ×100=37%.
Answer: Compressed=540 MB, Encrypted=567 MB, Reduction=37%. Presentation layer provides compression and encryption.

# Question 7

Given: Data=3 GB=3000 MB, Request size=3 MB, Throughput=120 Mbps.
Requests=3000/3=1000.
Transmission time= (3000×8)/120=200 s.
Answer: 1000 requests and 200 seconds. Application layer provides FTP services, authentication and user interaction.

# Question 8

Given: App=4 ms, Transport=3 ms, Network=5 ms, Data Link=2 ms, Physical=1 ms, Propagation=18 ms, Transmission=12 ms.
Total delay=4+3+5+2+1+18+12=45 ms.
Answer: End-to-end delay=45 ms. Each OSI layer contributes processing and communication functions.

# Question 9

Given: Useful data=80 MB=80,000,000 bytes, Frame size=1600 bytes, Overhead=80 bytes.
Useful payload/frame=1520 bytes.
Frames=80,000,000/1520≈52,632.
Overhead=52,632×80=4,210,560 bytes.
Efficiency= (80,000,000/(80,000,000+4,210,560)) ×100≈95.0%.
Answer: 52,632 frames, 4,210,560 bytes overhead, 95% efficiency.

# Question 10

Given: Medical record=240 MB, Compression=30%, Segment size=1400 bytes, Overhead=60 bytes/frame.
Compressed size=240×0.70=168 MB.
Segments=168,000,000/1400=120,000.
Overhead=120,000×60=7,200,000 bytes=7.2 MB.
Final transmitted data=168+7.2=175.2 MB.
Answer: Compressed size=168 MB, Segments=120,000, Overhead=7.2 MB, Final data=175.2 MB. Multiple OSI layers cooperate through compression, segmentation, framing and reliable delivery.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, well-explaine d answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structure d answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |

| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| --- | --- | --- | --- | --- | --- |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |