# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

NAME: P. HEMANTH REDDY

Reg.No:192312173

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

## ANALYTICAL PROBLEM SOLVING

- A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a

/28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

- A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet

range. Also, determine the number of usable host addresses available in each subnet.

- An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin →

/28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

- A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

- A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

ANSWERS

1. University Network Subnetting

Given

- Network Block = 192.168.20.0/24

- Lab A = /26

- Lab B = /27

- Lab C = /28

Formula

Usable Hosts = 2n-2

where n = Host Bits

Lab A (/26)

Subnet Mask = 255.255.255.192

Host bits = 32 − 26 = 6

Usable Hosts = 26-2=62

Block Size = 64

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.20.0 |
| First Host | 192.168.20.1 |
| Last Host | 192.168.20.62 |
| Broadcast Address | 192.168.20.63 |
| Usable Hosts | 62 |

Lab B (/27)

Subnet Mask = 255.255.255.224

Host bits = 5

Usable Hosts = 25-2=30

Next available subnet starts after Lab A.

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.20.64 |
| First Host | 192.168.20.65 |
| Last Host | 192.168.20.94 |
| Broadcast Address | 192.168.20.95 |
| Usable Hosts | 30 |

Lab C (/28)

Subnet Mask = 255.255.255.240

Host bits = 4

Usable Hosts = 24-2=14

Next available subnet starts after Lab B.

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.20.96 |
| First Host | 192.168.20.97 |
| Last Host | 192.168.20.110 |
| Broadcast Address | 192.168.20.111 |
| Usable Hosts | 14 |

2. Subnet Analysis of Two Systems

Given

System 1:

- IP = 192.168.1.130

- Mask = 255.255.255.192 (/26)

System 2:

- IP = 192.168.1.190

- Mask = 255.255.255.192 (/26)

Step 1: Determine Subnet Ranges

For /26:

Block Size = 64

Subnets:

- 192.168.1.0 – 63

- 192.168.1.64 – 127

- 192.168.1.128 – 191

- 192.168.1.192 – 255

System 1

IP = 192.168.1.130

Falls in:

192.168.1.128 – 191

Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.1.128 |
| First Host | 192.168.1.129 |
| Last Host | 192.168.1.190 |
| Broadcast | 192.168.1.191 |
| Usable Hosts | 62 |

✔ IP 192.168.1.130 is valid in this subnet.

System 2

IP = 192.168.1.190

Also falls in:

192.168.1.128 – 191

Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.1.128 |
| First Host | 192.168.1.129 |
| Last Host | 192.168.1.190 |
| Broadcast | 192.168.1.191 |
| Usable Hosts | 62 |

✔ IP 192.168.1.190 is valid in this subnet.

Conclusion

Both systems belong to the same subnet.

| System | Network Address | Host Range | Broadcast |
| --- | --- | --- | --- |
| System 1 | 192.168.1.128 | 192.168.1.129–190 | 192.168.1.191 |
| System 2 | 192.168.1.128 | 192.168.1.129–190 | 192.168.1.191 |

Usable Hosts = 62

3. Departmental Subnet Allocation

Given

Network Block = 172.16.0.0/16

Departments:

- HR → /25

- Sales → /26

- IT → /27

- Admin → /28

HR Department (/25)

Mask = 255.255.255.128

Usable Hosts:

27-2=126

| Parameter | Value |
| --- | --- |
| Network | 172.16.0.0 |
| Host Range | 172.16.0.1 – 172.16.0.126 |
| Broadcast | 172.16.0.127 |
| Usable Hosts | 126 |

Sales Department (/26)

Starts after HR.

| Parameter | Value |
| --- | --- |
| Network | 172.16.0.128 |
| Host Range | 172.16.0.129 – 172.16.0.190 |
| Broadcast | 172.16.0.191 |
| Usable Hosts | 62 |

IT Department (/27)

| Parameter | Value |
| --- | --- |
| Network | 172.16.0.192 |
| Host Range | 172.16.0.193 – 172.16.0.222 |
| Broadcast | 172.16.0.223 |
| Usable Hosts | 30 |

Admin Department (/28)

| Parameter | Value |
| --- | --- |
| Network | 172.16.0.224 |
| Host Range | 172.16.0.225 – 172.16.0.238 |
| Broadcast | 172.16.0.239 |
| Usable Hosts | 14 |

Remaining Unused Address Range

Allocated range ends at:

172.16.0.239

Remaining range:

172.16.0.240 – 172.16.255.255

Summary

| Department | Network | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- |
| HR | 172.16.0.0/25 | 172.16.0.1–126 | 172.16.0.127 | 126 |
| Sales | 172.16.0.128/26 | 172.16.0.129–190 | 172.16.0.191 | 62 |
| IT | 172.16.0.192/27 | 172.16.0.193–222 | 172.16.0.223 | 30 |
| Admin | 172.16.0.224/28 | 172.16.0.225–238 | 172.16.0.239 | 14 |

4. Subnetting Using /27

Given

Network = 172.16.10.0/24

Subnet Mask = /27

Total Number of Subnets

Borrowed Bits

= 27 − 24

= 3

Total Subnets

= 23=8

Hosts per Subnet

Host Bits

= 32 − 27

= 5

Usable Hosts

= 25-2=30

Subnet List

| Subnet | Range |
| --- | --- |
| 172.16.10.0 | 0–31 |
| 172.16.10.32 | 32–63 |
| 172.16.10.64 | 64–95 |
| 172.16.10.96 | 96–127 |
| 172.16.10.128 | 128–159 |
| 172.16.10.160 | 160–191 |
| 172.16.10.192 | 192–223 |
| 172.16.10.224 | 224–255 |

Determine Subnet of 172.16.10.78

78 falls between:

64 and 95

Therefore:

Network Address

172.16.10.64

Broadcast Address

172.16.10.95

Valid Host Range

172.16.10.65 – 172.16.10.94

Check IP 172.16.10.95

172.16.10.95 is the broadcast address.

Therefore:

❌ Not a usable host.

✔ It belongs to the same subnet but represents the broadcast address.

Final Answer

- Total Subnets = 8

- Usable Hosts/Subnet = 30

- IP 172.16.10.78 belongs to subnet 172.16.10.64/27

- Network Address = 172.16.10.64

- Broadcast Address = 172.16.10.95

- Host Range = 172.16.10.65–172.16.10.94

- 172.16.10.95 is the broadcast address, not a host address.

5. IPv6 Addressing

Given

IPv6 Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed Form

Rules:

- Remove leading zeros.

- Replace longest sequence of consecutive 0000 blocks with ::

Compressed Address:

2001:db8::ff00:42:8329

Expanded Form

Expanding back:

2001:0db8:0000:0000:0000:ff00:0042:8329

Network Prefix (/64)

First 64 bits represent the network.

Network Prefix:

2001:0db8:0000:0000::/64

Compressed:

2001:db8::/64

Number of Host Addresses

Host Bits

= 128 − 64

= 64

Possible Addresses

= 264

= 18,446,744,073,709,551,616

(approximately 18.4 quintillion addresses)

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formu la with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |