## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

NAME: P.HEMANTHREDDY
Reg.No:192312173

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine	the	network	address	and	broadcast	address	for	each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26 Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine	the	network	address	and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

ANSWERS

1. Smart Campus Network Design

Given

Network Block: 192.168.100.0/24

Departments:

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

School of Computing (/25)

Subnet Mask = 255.255.255.128

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.100.0 |
| First Host | 192.168.100.1 |
| Last Host | 192.168.100.126 |
| Broadcast Address | 192.168.100.127 |
| Usable Hosts | 126 |

School of Electronics (/26)

Next available subnet after /25

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.100.128 |
| First Host | 192.168.100.129 |
| Last Host | 192.168.100.190 |
| Broadcast Address | 192.168.100.191 |
| Usable Hosts | 62 |

School of Mechanical Engineering (/27)

Next available subnet after /26

Network Details

|  | Value |
| --- | --- |
| Network Address | 192.168.100.192 |
| First Host | 192.168.100.193 |
| Last Host | 192.168.100.222 |
| Broadcast Address | 192.168.100.223 |
| Usable Hosts | 30 |

Remaining Unused Address Range

Allocated addresses end at:

192.168.100.223

Unused Range:

192.168.100.224 – 192.168.100.255

Final Summary

| Department | Subnet | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- |
| Computing | 192.168.100.0/25 | 192.168.100.1 – 126 | 192.168.100.127 | 126 |
| Electronics | 192.168.100.128/26 | 192.168.100.129 – 190 | 192.168.100.191 | 62 |
| Mechanical | 192.168.100.192/27 | 192.168.100.193 – 222 | 192.168.100.223 | 30 |

2. Multinational Software Company

Given

Network Block = 10.10.0.0/16

Departments:

- Software Development → /24

- Quality Assurance → /25

- Human Resources → /26

- Executive Management → /27

Software Development (/24)

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 10.10.0.0 |
| First Host | 10.10.0.1 |
| Last Host | 10.10.0.254 |
| Broadcast | 10.10.0.255 |
| Usable Hosts | 254 |

Quality Assurance (/25)

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 10.10.1.0 |
| First Host | 10.10.1.1 |
| Last Host | 10.10.1.126 |
| Broadcast | 10.10.1.127 |
| Usable Hosts | 126 |

Human Resources (/26)

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 10.10.1.128 |
| First Host | 10.10.1.129 |
| Last Host | 10.10.1.190 |
| Broadcast | 10.10.1.191 |
| Usable Hosts | 62 |

Executive Management (/27)

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 10.10.1.192 |
| First Host | 10.10.1.193 |
| Last Host | 10.10.1.222 |
| Broadcast | 10.10.1.223 |
| Usable Hosts | 30 |

Remaining Unused Range

Allocated addresses end at:

10.10.1.223

Unused range:

10.10.1.224 – 10.10.255.255

Summary

| Department | Network | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- |
| Software Development | 10.10.0.0/24 | 10.10.0.1–254 | 10.10.0.255 | 254 |
| QA | 10.10.1.0/25 | 10.10.1.1–126 | 10.10.1.127 | 126 |
| HR | 10.10.1.128/26 | 10.10.1.129–190 | 10.10.1.191 | 62 |
| Executive | 10.10.1.192/27 | 10.10.1.193–222 | 10.10.1.223 | 30 |

3. Smart Hospital IPv6 Migration

Given

IPv6 Address:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Compressed Notation

Remove leading zeros and consecutive zero blocks.

2001:db8:abcd::/64

Expanded Notation

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Network Prefix (/64)

First 64 bits represent the network.

Network Prefix:

2001:0db8:abcd:0000::/64

Compressed Form:

2001:db8:abcd:0::/64

Host Portion

Host Bits:

128 − 64 = 64

Total Host Addresses

264
18,446,744,073,709,551,616

Approximately 18.4 Quintillion Addresses

Final Answer

| Parameter | Value |
| --- | --- |
| Original Address | 2001:0db8:abcd:0000:0000:0000:0000:0000/64 |
| Compressed Form | 2001:db8:abcd::/64 |
| Expanded Form | 2001:0db8:abcd:0000:0000:0000:0000:0000/64 |
| Network Prefix | 2001:db8:abcd::/64 |
| Host Bits | 64 |
| Total Hosts | 18,446,744,073,709,551,616 |

4. ISP Router Forwarding Scenario

Given Routing Table

Destination Network

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

Packet Destination:

192.168.2.45

Destination Subnet

Subnet Mask:

255.255.255.0 (/24)

Network Address:

192.168.2.45 AND 255.255.255.0

= 192.168.2.0

Therefore packet belongs to:

192.168.2.0/24

Matching Routing Entry

192.168.2.0/24

Next-Hop Interface

Router forwards packet through the interface connected to:

192.168.2.0/24 Network

(For example Interface G0/1 if configured.)

Default Route

If destination does not match any entry:

Default Route:

0.0.0.0/0

Example:

- Destination Network = 0.0.0.0/0

- Next Hop = ISP Gateway

Final Answer

| Task | Result |
| --- | --- |
| Destination Subnet | 192.168.2.0/24 |
| Matching Route | 192.168.2.0/24 |
| Forwarding Interface | Interface connected to 192.168.2.0/24 |
| Default Route | 0.0.0.0/0 |

5. Interconnected Office LANs

Given

Administration LAN = 192.168.10.0/26

Finance LAN = 192.168.10.64/26

Router Interfaces:

- Administration Gateway = 192.168.10.1

- Finance Gateway = 192.168.10.65

Administration LAN

Subnet Mask = /26

Block Size = 64

Range:

0 – 63

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.10.0 |
| First Host | 192.168.10.1 |
| Last Host | 192.168.10.62 |
| Broadcast Address | 192.168.10.63 |

Valid Host Range

192.168.10.1 – 192.168.10.62

Three Computer IP Addresses

- PC-A1 → 192.168.10.2

- PC-A2 → 192.168.10.3

- PC-A3 → 192.168.10.4

Default Gateway

192.168.10.1

Finance LAN

Range:

64 – 127

Network Details

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.10.64 |
| First Host | 192.168.10.65 |
| Last Host | 192.168.10.126 |
| Broadcast Address | 192.168.10.127 |

Valid Host Range

192.168.10.65 – 192.168.10.126

Three Computer IP Addresses

- PC-F1 → 192.168.10.66

- PC-F2 → 192.168.10.67

- PC-F3 → 192.168.10.68

Default Gateway

192.168.10.65

Final Summary

| LAN | Network Address | Host Range | Broadcast Address | Gateway |
| --- | --- | --- | --- | --- |
| Administration | 192.168.10.0/26 | 192.168.10.1–62 | 192.168.10.63 | 192.168.10.1 |
| Finance | 192.168.10.64/26 | 192.168.10.65–126 | 192.168.10.127 | 192.168.10.65 |

Suggested PCs

Administration LAN

- 192.168.10.2

- 192.168.10.3

- 192.168.10.4

Finance LAN

- 192.168.10.66

- 192.168.10.67

- 192.168.10.68

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning ) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned , logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |