# SIMATS ENGINEERING

### ASSESSMENT TOOL 3

#### COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

#### NAME:P.HEMANTH REDDY

#### REG.NO:192312173

#### COURSE OUTCOME COVERED

##### CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

#### QUESTIONS: 5	Total Marks:25

Annexure A

## Comparative Analysis

- Star topology is generally more expensive than bus topology because it requires a central device such as a switch or hub and more cabling to connect each node individually. In contrast, bus topology uses a single backbone cable, making it low-cost and easier to install. However, in terms of reliability, star topology is superior because failure of one node does not affect others, whereas a failure in the main bus cable can bring down the entire network. Performance is also better in star topology since data transmission is managed through a central device, reducing collisions, while bus topology suffers from performance degradation as more devices are added due to increased data traffic on the shared medium.

- Mesh topology offers very high fault tolerance because every node is connected to multiple other nodes, providing multiple paths for data transmission. If one link fails, data can still be routed through alternate paths without disrupting communication. In contrast, star topology has moderate fault tolerance; while failure of individual nodes does not affect the network, failure of the central hub or switch can cause the entire network to stop functioning. Therefore, mesh topology is more robust and reliable in critical environments compared to star topology.

- Bus topology is simple and easy to install because it uses a single backbone cable with minimal connections, making it suitable for small networks. On the other hand, mesh topology is highly complex to install as each node must be connected to every other node, requiring a large number of cables and ports. This complexity increases significantly as the number of devices grows, making mesh topology time-consuming and costly to implement compared to the straightforward setup of bus topology.

- Hybrid topology is highly scalable because it combines two or more different topologies (such as star, mesh, or bus), allowing the network to expand flexibly according to organizational needs. New segments can be added without affecting the existing network structure. In comparison, star topology is also scalable but to a limited extent, as it depends on the capacity of the central device (switch or hub). Once the central device reaches its port limit, expansion requires additional hardware. Thus, hybrid topology provides greater scalability than star topology.

- In terms of maintenance, star topology is relatively easy to manage because issues can be quickly identified and isolated to a specific node or cable. Mesh topology, while highly reliable, is difficult to maintain due to its complex structure and large number of connections, making troubleshooting time-consuming. Bus topology is harder to maintain than star because faults in the backbone cable can disrupt the entire network and are difficult to locate. Hybrid topology has moderate maintenance complexity; while it benefits from flexibility, managing multiple combined topologies requires careful monitoring and skilled administration.

Answers:

- Star Topology vs Bus Topology – Cost, Reliability, and Performance

| Factor | Star Topology | Bus Topology |
| --- | --- | --- |
| Cost | Higher because it requires a central switch and individual cables | Lower because it uses a common backbone cable |
| Installation | Relatively easy but requires more cabling | Simple with fewer cables |
| Reliability | High; failure of one cable affects only one device | Low; backbone failure can affect the entire network |
| Performance | Better in switched Ethernet because each device has a dedicated link | Decreases as more devices share the medium |
| Troubleshooting | Easy to identify the faulty device or cable | Difficult because faults in the backbone can affect many devices |
| Scalability | Good, depending on switch port capacity | Poor for larger networks |

Analysis: Star topology has a higher initial cost but provides better reliability, performance, and manageability. Bus topology is cheaper but is more suitable for small or legacy networks.

2. Mesh Topology vs Star Topology – Fault Tolerance

| Factor | Mesh Topology | Star Topology |
| --- | --- | --- |
| Fault Tolerance | Very high | Moderate |
| Redundancy | Multiple communication paths | Usually one path through the central switch |
| Single Point of Failure | Very low in a properly redundant mesh | Central switch can become a single point of failure |
| Reliability | Excellent | Good |
| Cost | Very high | Moderate |
| Complexity | High | Low |
| Typical Use | Banks, data centers, critical networks | Offices, schools, small businesses |

Analysis: Mesh topology provides superior fault tolerance because alternative paths are available. Star topology is simpler and cheaper, but failure of the central switch can interrupt communication for all connected devices. Redundant switches can reduce this weakness.

3. Bus Topology vs Mesh Topology – Installation Complexity

| Factor | Bus Topology | Mesh Topology |
| --- | --- | --- |
| Cabling | Single backbone | Multiple links between nodes |
| Installation | Simple | Complex |
| Number of Connections | Low | Very high |
| Cost | Low | High |
| Maintenance | Relatively difficult when backbone faults occur | Complex because of many links |
| Scalability | Poor | Technically high but expensive |
| Suitable For | Small/legacy networks | Critical and redundant networks |

For a full mesh, the number of links required is:

where is the number of devices.

For example, with 5 devices:

With 10 devices:

Therefore, the number of connections increases rapidly as more devices are added.

Analysis: Bus topology is much easier and cheaper to install, while mesh topology provides greater redundancy at the cost of significantly higher complexity and infrastructure requirements.

4. Hybrid Topology vs Star Topology – Scalability

| Factor | Hybrid Topology | Star Topology |
| --- | --- | --- |
| Scalability | Very high | Good |
| Expansion | New topology segments can be added | Limited by switch capacity |
| Flexibility | Very high | Moderate |
| Complexity | Higher | Lower |
| Cost | Depends on the topologies used | Moderate |
| Management | More complex | Easier |
| Suitable For | Large organizations and campuses | Offices and small networks |

Analysis: Hybrid topology is more flexible because different topology types can be combined according to specific requirements. For example, a university can use a mesh or ring backbone, star topology within buildings, and VLAN-based access networks.

Star topology is simpler, but expansion eventually requires additional switches or hardware when the existing switch runs out of ports.

5. Maintenance Comparison

| Topology | Maintenance Level | Main Reason |
| --- | --- | --- |
| Star | Easy | Individual failures are easy to isolate |
| Mesh | Difficult | Large number of connections |
| Bus | Moderate/Difficult | Backbone failure can affect the whole network |
| Hybrid | Moderate/High | Multiple topology types must be managed |

Star topology:
A faulty cable or device can usually be identified quickly without affecting the other nodes.

Mesh topology:
Although highly reliable, it has many connections. Network administrators must monitor and troubleshoot a larger number of links.

Bus topology:
A fault in the backbone can affect multiple devices, and locating the exact fault can be difficult.

Hybrid topology:
Maintenance depends on the topologies used. It provides flexibility but requires skilled network administration and monitoring.

There is no single topology that is best for every situation. Star topology is preferred for offices because it provides good performance and easy maintenance. Bus topology is inexpensive and simple but has poor scalability and fault tolerance. Mesh topology provides the highest reliability and redundancy, making it suitable for critical networks such as banking and data centers. Hybrid topology offers the greatest flexibility and scalability and is therefore well suited to large campus and enterprise networks.

#### RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 15% | Demonstrates complete and in-depth understanding of all concepts being compared | Good understanding with minor gaps | Basic understanding; some concepts unclear | Poor understanding of concepts |
| Comparison Depth | 20% | Provides detailed, insightful, and meaningful comparisons across multiple aspects | Adequate comparison with relevant points | Limited comparison; lacks depth | Minimal or incorrect comparison |
| Criteria Selection | 10% | Selects highly relevant and appropriate comparison parameters | Mostly relevant criteria | Some irrelevant or missing criteria | Poor or no criteria selection |
| Analytical Skills | 15% | Strong analysis with logical reasoning and clear justification | Good analysis with some justification | Basic analysis with weak reasoning | No clear analysis |
| Organization & Structure | 10% | Well-organized, clear, and logically structured | Generally organized | Somewhat disorganized | Poorly structured |
| Use of Examples / Evidence | 10% | Uses strong, relevant examples or data to support comparison | Uses some examples | Limited examples | No supporting examples |
| Clarity of Presentation | 10% | Very clear, concise, and easy to understand | Mostly clear | Somewhat unclear | Difficult to understand |
| Conclusion & Insights | 10% | Insightful conclusion with strong justification | Clear conclusion | Basic conclusion | No or weak conclusion |