# SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

NAME:P HEMANTH REDDY

REG.NO:192312173

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

## Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile

devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

ANSWER

1. Network Design for a 20-Person Startup

Recommended Topology: Star Topology

A star topology is the most suitable choice for a 20-person startup because all computers and network devices are connected to a central managed switch.

Network Architecture

Internet

|

[Router / Firewall]

|

[24-Port Managed

Switch]

/  /  |  \  \

/  /   |   \  \

PC1 PC2  PC3  PC4  ...

\

PC20

|

[Wi-Fi Access Point]

Components

- 24-port Layer 2/Layer 3 managed switch

- Cat6 UTP cables

- Router/firewall

- Wi-Fi access point

- 20 employee workstations

- Printers

- VoIP phones

- DHCP and NAT services

Working

Each workstation has a dedicated connection to the central switch. The switch forwards Ethernet frames between devices, while the router/firewall provides Internet access, DHCP, NAT, and security.

Why Star Topology?

The major advantage is fault isolation. If the cable or network interface of one workstation fails, only that workstation is affected. The remaining employees can continue working normally.

Advantages

- Easy to install and manage.

- Easy fault detection.

- Failure of one cable does not affect other devices.

- Easy to add new employees.

- Supports high-speed Gigabit Ethernet.

- Centralized network monitoring is possible.

Conclusion

For a 20-person startup, star topology provides a good balance of cost, reliability, scalability, and easy maintenance.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

ANSWER

Recommended Topology: Partial Mesh / Full Mesh

A bank requires very high availability because network downtime can affect financial transactions and critical services. Therefore, a redundant partial-mesh topology is the most practical choice.

Network Architecture

Internet 1

|

[Router 1]

|

+-----+-----+

|           |

[Core SW1]===== [Core SW2]

||           ||

||           ||

[Core SW3]===== [Firewall]

|             |

Servers        Internet 2

The multiple links provide alternative paths.

Components

- Redundant core switches

- Dual enterprise routers

- Firewalls

- Fiber-optic links

- Servers

- UPS systems

- RAID storage

- Redundant power supplies

Working

Multiple network paths exist between critical devices. If one link fails, routing protocols such as OSPF or BGP can select another available path.

For example:

Normal:

Core A -------- Core B

|               |

+---- Server ---+

If Core A → Core B link fails:

Core A

|

Core C -------- Core B

Traffic can continue through the alternate path.

Why Mesh?

The major advantage is redundancy. There is no dependence on a single communication path.

Advantages

- Very high availability.

- Multiple backup paths.

- Automatic rerouting.

- Suitable for mission-critical applications.

- Fiber provides high bandwidth.

- Supports redundant Internet connections.

Limitation

A full mesh requires a large number of links and ports, making it expensive and complex. Therefore, banks generally use partial mesh rather than connecting every device directly to every other device.

Conclusion

A partial-mesh core with redundant switches, routers, firewalls, and fiber links provides the reliability required for banking networks.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8-or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

ANSWER

Recommended Topology: Bus Topology

For a small classroom with very limited budget and light network traffic, a bus topology can be considered as a low-cost educational design.

Basic Architecture

50Ω Terminator                              50Ω Terminator

|                                             |

|=============================================|

|          |          |          |

PC-1       PC-2       PC-3       PC-4

In the traditional Ethernet bus arrangement, computers share a common coaxial backbone.

Components

- RG-58 coaxial cable

- BNC connectors

- T-connectors

- 50-ohm terminators

- Network interface cards

- Computers

Working

All computers share the same physical communication medium. When one computer transmits, the signal travels along the common bus.

Advantages

- Low installation cost.

- Less cable required.

- No central switch is required in a classic bus implementation.

- Simple architecture.

- Suitable for small educational demonstrations.

Limitations

- A backbone cable failure can affect the entire network.

- Collisions can occur in shared-medium Ethernet.

- Troubleshooting can be difficult.

- Network performance decreases as traffic increases.

- Bus Ethernet is obsolete for modern production LANs.

Important Practical Note

Modern Ethernet networks generally use star topology with switches, even in inexpensive classrooms. A modern low-cost classroom would normally use an 8- or 16-port unmanaged switch and Cat5e/Cat6 cables rather than a physical coaxial bus.

Conclusion

A bus topology is useful as a low-cost educational or legacy-network example, but a switched star topology is the better choice for an actual modern classroom.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for

redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

ANSWER

Network Design for a Large University Campus

Recommended Topology: Hybrid Topology

A university has thousands of users spread across multiple buildings. Therefore, a single topology is not sufficient. A hybrid topology combining star, ring/mesh, and hierarchical structures is appropriate.

Network Architecture

CAMPUS CORE

+----------------------+

|                      |

[Core SW1]==============[Core SW2]

||                      ||

||                      ||

[Core SW3]==============[Core SW4]

/          \          /

/            \        /

Building A     Building B    Building C

|               |            |

Distribution     Distribution  Distribution

Switch            Switch       Switch

/ | \             / | \         / | \

SW SW SW          SW SW SW      SW SW SW

|  |  |            |  |  |        |  |  |

PCs PCs APs         PCs PCs APs     PCs PCs APs

Three-Layer Architecture

1. Core Layer

Provides high-speed connectivity between buildings.

- Fiber optic

- High-speed multilayer switches

- Redundant links

- 10 Gbps/40 Gbps or higher depending on requirements

2. Distribution Layer

Connects individual buildings to the campus core.

- Distribution switches

- Dual fiber uplinks

- Routing

- VLAN aggregation

- Security policies

3. Access Layer

Connects end devices.

- PCs

- Printers

- IP phones

- Wireless access points

- IoT devices

Advantages

- Highly scalable.

- High-speed backbone.

- Redundant connections.

- Easy building expansion.

- Centralized management.

- Supports wired and wireless users.

- Failure of one backbone link does not necessarily isolate a building.

Example

If a new engineering building is added:

New Building

|

Distribution Switch

||

Dual Fiber Links

||

Campus Core

Existing buildings can continue operating while the new building is integrated.

Conclusion

A hybrid hierarchical topology provides the university with scalability, redundancy, performance, and easy management.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the

access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

ANSWER

Network Design for a Company with Multiple Departments

Recommended Design: Hierarchical VLAN-Based Network

For departments such as HR, Finance, Sales, and IT, VLAN-based segmentation with Layer 2 access switches and a Layer 3 core switch is recommended.

Network Architecture

Internet

|

[Router / Firewall]

|

[Layer 3 Switch]

/      |       \

/       |        \

VLAN 10   VLAN 20   VLAN 30

HR       Finance     Sales

|          |          |

[L2 Switch] [L2 Switch] [L2 Switch]

/ \          / \          / \

PCs PCs      PCs PCs      PCs PCs

VLAN 40

IT

|

[L2 Switch]

/   \

PCs   PCs

VLAN Configuration

| Department | VLAN | Example Network |
| --- | --- | --- |
| HR | VLAN 10 | 192.168.10.0/24 |
| Finance | VLAN 20 | 192.168.20.0/24 |
| Sales | VLAN 30 | 192.168.30.0/24 |
| IT | VLAN 40 | 192.168.40.0/24 |

Working

Each department is assigned a separate VLAN.

For example:

HR PC → VLAN 10

Finance PC → VLAN 20

Sales PC → VLAN 30

IT PC → VLAN 40

Devices within the same VLAN can communicate at Layer 2.

Communication between different VLANs requires inter-VLAN routing, which can be performed by a Layer 3 switch or router.

Security

VLAN segmentation helps prevent unnecessary direct Layer 2 communication between departments. Additional firewall or ACL rules can control which departments are allowed to communicate.

For example:

HR  → Finance   ALLOW

Sales → Finance BLOCK

IT → All       ALLOW

The exact rules depend on the organization's security policy.

Advantages

- Improved network security.

- Reduced broadcast traffic.

- Better network organization.

- Easier IP address management.

- Easy department expansion.

- Inter-VLAN routing provides controlled communication.

- Redundant links can improve reliability.

Conclusion

A hierarchical VLAN-based architecture is an efficient solution for a multi-department company because it provides security, scalability, performance, and centralized management.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevan t solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solvin g Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documente d | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and | Good collaboration | Limited participation | No collaboration or contribution |

| Criteria | W4 eightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
|  |  | contribution (if team-based) |  |  |  |