# Computer Networks (CSA0716)

# CO 1Assessment Tool 1

# Question 1

Given: File=150 MB, Segment size=1500 bytes, Bandwidth=50 Mbps, DL overhead=40 bytes/frame.
Segments=150,000,000/1500=100,000.
Frames=100,000.
Total overhead=100,000×40=4,000,000 bytes=4 MB.
Total transmitted data=154 MB.
Transmission time=(154,000,000×8)/50,000,000=24.64 s.
Answer: 100,000 segments, 100,000 frames, 4 MB overhead, 24.64 s. Transport layer provides segmentation, sequencing and error recovery. Data Link layer provides framing, error detection and reliable node-to-node delivery.

# Question 2

Given: Window size=6, Frame size=1024 bytes, Total frames=48.
Windows required=48/6=8.
One frame lost in each window.
Retransmissions=8×1=8.
Answer: 8 windows and 8 retransmissions. Flow control prevents congestion and receiver overflow.

# Question 3

Given: Packet=12,000 bytes, MTU=1500 bytes, IP header=20 bytes.
Payload/fragment=1500−20=1480 bytes.
Fragments=ceil(12000/1480)=9.
Header overhead=9×20=180 bytes.
Answer: 9 fragments and 180 bytes overhead. Network layer performs routing, fragmentation and reassembly.

# Question 4

Given: Window size=6, Total frames=48.
Windows=48/6=8.
Retransmissions=8.
Answer: 8 transmission windows and 8 retransmissions. Flow control balances sender and receiver speeds.

# Question 5

Given: File=600 MB, Checkpoint every 60 MB, Failure after 390 MB.
Checkpoints before failure=360/60=6.
Last checkpoint=360 MB.
Data to retransmit=390−360=30 MB.
Answer: 6 checkpoints and 30 MB retransmission. Session layer synchronization enables recovery from failures.

# Question 6

Given: Original file=900 MB, Compression=40%, Encryption overhead=5%.
Compressed size=900×0.60=540 MB.
Encrypted size=540×1.05=567 MB.
Reduction=((900−567)/900)×100=37%.
Answer: Compressed=540 MB, Encrypted=567 MB, Reduction=37%. Presentation layer provides compression and encryption.

# Question 7

Given: Data=3 GB=3000 MB, Request size=3 MB, Throughput=120 Mbps.
Requests=3000/3=1000.
Transmission time=(3000×8)/120=200 s.
Answer: 1000 requests and 200 seconds. Application layer provides FTP services, authentication and user interaction.

# Question 8

Given: App=4 ms, Transport=3 ms, Network=5 ms, Data Link=2 ms, Physical=1 ms, Propagation=18 ms, Transmission=12 ms.
Total delay=4+3+5+2+1+18+12=45 ms.
Answer: End-to-end delay=45 ms. Each OSI layer contributes processing and communication functions.

# Question 9

Given: Useful data=80 MB=80,000,000 bytes, Frame size=1600 bytes, Overhead=80 bytes.
Useful payload/frame=1520 bytes.
Frames=80,000,000/1520≈52,632.
Overhead=52,632×80=4,210,560 bytes.
Efficiency=(80,000,000/(80,000,000+4,210,560))×100≈95.0%.
Answer: 52,632 frames, 4,210,560 bytes overhead, 95% efficiency.

# Question 10

Given: Medical record=240 MB, Compression=30%, Segment size=1400 bytes, Overhead=60 bytes/frame.
Compressed size=240×0.70=168 MB.
Segments=168,000,000/1400=120,000.
Overhead=120,000×60=7,200,000 bytes=7.2 MB.
Final transmitted data=168+7.2=175.2 MB.
Answer: Compressed size=168 MB, Segments=120,000, Overhead=7.2 MB, Final data=175.2 MB. Multiple OSI layers cooperate through compression, segmentation, framing and reliable delivery.