# SIMATS ENGINEERING

### ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3	Total Marks:30

Annexure A

## Constraint-Based Problem Solving

- A multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

ANSWERS

1. Remote Rural Branch Office

Problem Analysis

The company needs:

- Internet access in a rural location

- Cloud applications

- Video conferencing

- Secure connection to headquarters

- Limited connectivity budget

- High reliability

- Future scalability

Recommended Solution

A 4G/5G cellular broadband connection as the primary link, with a satellite connection as a backup where cellular coverage is unreliable, is a practical design.

Network Architecture

4G/5G Internet

|

[5G/4G Router]

|

[Firewall]

|

[Managed Switch]

/       |       \

PCs      VoIP      Wi-Fi AP

|

Headquarters

VPN Tunnel

Backup:

Satellite Internet

|

Backup WAN Link

|

Firewall

Recommended Devices

| Device | Purpose |
| --- | --- |
| 4G/5G Router | Primary Internet connection |
| Directional/high-gain antenna | Improves cellular reception where needed |
| Firewall/VPN gateway | Security and headquarters VPN |
| Managed Gigabit switch | Connects local devices |
| Wi-Fi 6/6E AP | Wireless connectivity |
| UPS | Maintains operation during short power failures |

Connectivity Recommendation

Primary: 4G/5G broadband.

Backup: Satellite broadband if affordable and available.

A dual-WAN firewall/router can automatically switch to the backup connection if the primary connection fails.

Security

The branch should use:

- Site-to-site IPsec VPN to headquarters

- Stateful firewall

- WPA3 for Wi-Fi

- Strong administrator authentication

- Separate guest Wi-Fi

- Regular firmware updates

- Network monitoring

Constraint Analysis

Bandwidth: 5G can provide substantially better performance than older cellular technologies where coverage is good. Video conferencing should be prioritized using QoS.

Reliability: Dual-WAN connectivity reduces dependence on a single Internet provider.

Cost: Using cellular broadband as the primary connection avoids the potentially high cost of dedicated rural leased lines.

Scalability: A business-grade firewall and managed switch can accommodate additional employees, access points, and IoT devices.

Final Recommendation

A 4G/5G primary connection + backup WAN + firewall + VPN + managed switch + Wi-Fi provides a practical balance between cost, reliability, security, and scalability.

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

ANSWER

University Wireless Network for 3,000+ Students

Problem Analysis

The university requires:

- More than 3,000 users

- Campus-wide coverage

- High simultaneous-user capacity

- Minimal interference

- Secure authentication

- Limited number of APs

- Controlled deployment cost

The major challenge is that coverage alone is not enough. Access points must also have sufficient capacity for the number of simultaneous users.

Recommended Design

Use centrally managed Wi-Fi 6/6E access points with a wireless controller or cloud-based management platform.

Proposed Architecture

Internet

|

[Core Router]

|

[Core L3 Switch]

|

[Wireless Controller]

/      |       \

/       |        \

AP-1     AP-2      AP-3

/|\       /|\       /|\

Students  Students   Students

Access points should be distributed among:

- Classrooms

- Laboratories

- Libraries

- Hostels

- Common areas

- Student activity areas

Access Point Placement

Instead of placing APs only at equal distances, deployment should be based on a site survey.

High-density areas should receive more AP capacity:

High-density:

Lecture halls

Libraries

Hostels

Laboratories

↓

More AP capacity

Lower-density areas can use fewer APs.

APs should preferably be mounted in suitable ceiling or wall locations with minimal obstruction.

Frequency Selection

2.4 GHz

Advantages:

- Longer range

- Better penetration

Disadvantages:

- Limited number of non-overlapping channels

- More interference

- Lower capacity

5 GHz

Advantages:

- More channels

- Higher performance

- Better for high-density environments

Therefore, 5 GHz should be the primary band for high-capacity student connectivity.

6 GHz

If Wi-Fi 6E-compatible devices are widely available and permitted by local regulations, 6 GHz can provide additional spectrum and reduce congestion.

Channel Planning

Adjacent APs should use carefully planned channels and transmit power.

Instead of:

AP1 = High Power

AP2 = High Power

AP3 = High Power

use controlled transmit power:

AP1 ---- Channel A

AP2 ---- Channel B

AP3 ---- Channel C

This reduces co-channel interference.

Authentication

For students and staff, use WPA2/WPA3-Enterprise with 802.1X authentication, preferably backed by a RADIUS service.

Example:

Student Device

|

WPA3-Enterprise

|

802.1X

|

RADIUS

|

Network Access

Guest users should receive a separate guest network with restricted access.

Network Segmentation

Separate networks can be created using VLANs:

| User | VLAN |
| --- | --- |
| Students | VLAN 10 |
| Faculty | VLAN 20 |
| Administration | VLAN 30 |
| Guests | VLAN 40 |
| IoT | VLAN 50 |

Balancing the Constraints

Coverage: APs are positioned using site-survey results.

Performance: 5 GHz/6 GHz is prioritized and high-density areas receive additional AP capacity.

Security: WPA2/WPA3-Enterprise, 802.1X, RADIUS, and VLAN segmentation protect users and institutional resources.

Cost: Instead of deploying excessive APs everywhere, AP density is increased only in areas with high user concentration.

Final Recommendation

A Wi-Fi 6/6E centrally managed network, with carefully planned AP placement, 5/6 GHz priority, 802.1X authentication, VLAN segmentation, and centralized monitoring provides a good balance between coverage, capacity, security, interference management, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

ANSWER

Secure Network for a Financial Institution

Problem Analysis

The financial institution must:

- Protect sensitive customer information

- Provide employees with controlled access

- Support online banking services

- Meet security and compliance requirements

- Maintain good network performance

- Control operational costs

A defense-in-depth architecture is recommended.

Proposed Network Architecture

Internet

|

[Edge Firewall]

|

[DMZ / WAF Zone]

/            \

Web Servers      Public Services

|

[Internal FW]

|

[Core L3 Switch]

/       |        \

/        |         \

VLAN 10   VLAN 20    VLAN 30

Users    Servers     Admin

|        |          |

PCs/VDI   Database   IT Systems

1. Network Segmentation

VLANs should separate different security zones.

Example:

| Network | VLAN | Purpose |
| --- | --- | --- |
| Employee | 10 | Normal employee systems |
| Finance | 20 | Financial operations |
| Servers | 30 | Application servers |
| Database | 40 | Sensitive customer data |
| Management | 50 | Network administration |
| Guest | 60 | Internet-only access |

Why Segmentation?

Without segmentation:

Employee PC → Database

could potentially provide an unnecessary attack path.

With segmentation:

Employee VLAN

|

Firewall

|

Access Policy

|

Database VLAN

Only explicitly authorized traffic is permitted.

2. Firewall Deployment

A layered firewall architecture is recommended.

Internet Firewall

Protects the organization from external attacks.

Internal Firewall

Controls traffic between sensitive network zones.

DMZ

Public-facing systems such as web applications should be isolated in a DMZ rather than placed directly on the internal network.

Internet

|

Firewall

|

DMZ

|

Internal Firewall

|

Internal Network

A web application firewall (WAF) can additionally protect suitable HTTP/HTTPS applications.

3. Authentication

Employees accessing sensitive systems should use:

Multi-Factor Authentication

Username + Password

+

Authenticator / Security Key

↓

Access

Use centralized identity management and role-based access control (RBAC).

For example:

HR Employee → HR applications

Finance Employee → Finance applications

IT Administrator → Network management

Users should receive only the permissions necessary for their jobs.

4. Intrusion Detection and Prevention

Deploy IDS/IPS at appropriate network boundaries.

IDS

Detects suspicious activity and generates alerts.

IPS

Can actively block or prevent detected malicious traffic.

Example:

Internet

|

Firewall

|

IDS/IPS

|

DMZ

|

Internal Network

Security monitoring should also collect:

- Firewall logs

- Authentication logs

- Server logs

- IDS/IPS alerts

- Network events

A centralized SIEM can correlate these events and assist security monitoring and incident response.

5. Security Controls

Additional controls should include:

- Endpoint protection/EDR

- Network access control where appropriate

- Encryption for sensitive communications

- Regular vulnerability scanning

- Patch management

- Secure backups

- Least-privilege access

- Administrative access through controlled management networks

- Continuous security monitoring

Constraint Analysis

Security

Network segmentation, firewalls, MFA, IDS/IPS, encryption, and least privilege provide multiple layers of protection.

Performance

High-speed Layer 3 switching can handle internal traffic efficiently, while firewall inspection is applied strategically to sensitive boundaries rather than unnecessarily inspecting every internal flow.

Compliance

Centralized logging, access controls, authentication, encryption, monitoring, and documented security policies help support applicable financial-sector regulatory and audit requirements.

Budget

Instead of deploying expensive security appliances everywhere, security controls should be prioritized around Internet gateways, sensitive servers, databases, privileged access, and critical applications.

Scalability

VLANs, centralized authentication, and policy-based security allow new employees, departments, and applications to be added without redesigning the entire network.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |